| Device | Interface |  | IP Address |  | Subnet Mask |  | Default Gateway |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1 | G0/0 |  | 209.165.200.225 |  | 255.255.255.248 |  | N/A |
| S0/0/0 (DCE) |  | 10.1.1.1 |  | 255.255.255.252 |  | N/A |  |
| R2 | S0/0/0 |  | 10.1.1.2 |  | 255.255.255.252 |  | N/A |
| S0/0/1 (DCE) |  | 10.2.2.2 |  | 255.255.255.252 |  | N/A |  |
| R3 | G0/1 |  | 172.16.3.1 |  | 255.255.255.0 |  | N/A |
| S0/0/1 |  | 10.2.2.1 |  | 255.255.255.252 |  | N/A |  |
| ASA | VLAN 1 (E0/1) |  | 192.168.1.1 |  | 255.255.255.0 |  | NA |
| ASA | VLAN 2 (E0/0) |  | 209.165.200.226 |  | 255.255.255.248 |  | NA |
| ASA | VLAN 3 (E0/2) |  | 192.168.2.1 |  | 255.255.255.0 |  | NA |
| DMZ Server | NIC |  | 192.168.2.3 |  | 255.255.255.0 |  | 192.168.2.1 |
| PC-B | NIC |  | 192.168.1.3 |  | 255.255.255.0 |  | 192.168.1.1 |

Scripts

ASA enable

!<Enter> for password conf t

hostname CCNAS-ASA

domain-name ccnasecurity.com

enable password ciscoenpa55

clock set 13:52:51 June 10 2015

interface vlan 1

nameif inside

ip address 192.168.1.1 255.255.255.0

security-level 100

interface vlan 2

nameif outside

ip address 209.165.200.226 255.255.255.248 s

ecurity-level 0

route outside 0.0.0.0 0.0.0.0 209.165.200.225

object network inside-net subnet 192.168.1.0 255.255.255.0

nat (inside,outside) dynamic interface

class-map inspection_default

match default-inspection-traffic

exit

policy-map global_policy

class inspection_default i

nspect icmp

exit

service-policy global_policy global

dhcpd address 192.168.1.5-192.168.1.36 inside

dhcpd dns 209.165.201.2 interface inside

dhcpd enable inside

username admin password adminpa55

aaa authentication ssh console LOCAL

crypto key generate rsa modulus 1024

no

ssh 192.168.1.0 255.255.255.0 inside ssh 172.16.3.3 255.255.255.255 outside ssh timeout 10 interface vlan 3

ip address 192.168.2.1 255.255.255.0

no forward interface vlan 1

nameif dmz

security-level 70

interface Ethernet0/2

switchport access vlan 3

object network dmz-server

host 192.168.2.3

nat (dmz,outside) static 209.165.200.227

access-list OUTSIDE-DMZ permit icmp any host 192.168.2.3

access-list OUTSIDE-DMZ permit tcp any host 192.168.2.3 eq 80

access-group OUTSIDE-DMZ in interface outside PC-B

-Change from static to DHCP addressing