!!! Script for R1 config t

license boot module c1900 technology-package securityk9

yes

end

copy running-config startup-config

reload

config t

access-list 110 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255

crypto isakmp policy 10

encryption aes 256

authentication pre-share

group 5

exit

crypto isakmp key vpnpa55 address 10.2.2.2

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

crypto map VPN-MAP 10 ipsec-isakmp

description VPN connection to R3

set peer 10.2.2.2 set transform-set VPN-SET

match address 110

exit

interface S0/0/0

crypto map VPN-MAP

!!! Script for R3 config t

access-list 110 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255 crypto isakmp policy 10

encryption aes 256 authentication

pre-share

group 5

crypto isakmp key vpnpa55 address 10.1.1.2

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

crypto map VPN-MAP 10 ipsec-isakmp

description VPN connection to R1

set peer 10.1.1.2

set transform-set VPN-SET

match address 110

exit

interface S0/0/1

crypto map VPN-MAP