Configure Routers: OSPF MD5 authentication.  NTP. to log messages to the syslog server.

1️⃣ Router0 Configuration

enable
configure terminal
hostname R0

interface g0/0
ip address 192.168.1.1 255.255.255.0
no shutdown

interface s0/1/0
ip address 192.168.2.1 255.255.255.252
clock rate 64000
no shutdown
ip ospf message-digest-key 1 md5 cisco
ip ospf authentication message-digest

router ospf 1
router-id 1.1.1.1
network 192.168.1.0 0.0.0.255 area 0
network 192.168.2.0 0.0.0.3 area 0

ntp server 192.168.1.10

logging 192.168.1.20
logging trap debugging
service timestamps log datetime msec

end
write memory

2️⃣ Router1 Configuration

enable
configure terminal
hostname R1

interface s0/1/0
ip address 192.168.2.2 255.255.255.252
no shutdown
ip ospf message-digest-key 1 md5 cisco
ip ospf authentication message-digest

interface s0/1/1
ip address 192.168.4.1 255.255.255.252
clock rate 64000
no shutdown
ip ospf message-digest-key 1 md5 cisco
ip ospf authentication message-digest

router ospf 1
router-id 2.2.2.2
network 192.168.2.0 0.0.0.3 area 0
network 192.168.4.0 0.0.0.3 area 0

ntp server 192.168.1.10

logging 192.168.1.20
logging trap debugging
service timestamps log datetime msec

end
write memory

3️⃣ Router2 Configuration

enable
configure terminal
hostname R2

interface g0/0
ip address 192.168.3.1 255.255.255.0
no shutdown

interface s0/1/1
ip address 192.168.4.2 255.255.255.252
no shutdown
ip ospf message-digest-key 1 md5 cisco
ip ospf authentication message-digest

router ospf 1
router-id 3.3.3.3
network 192.168.3.0 0.0.0.255 area 0
network 192.168.4.0 0.0.0.3 area 0

ntp server 192.168.1.10

logging 192.168.1.20
logging trap debugging
service timestamps log datetime msec

end
write memory

4️⃣ Switch0 Configuration (Servers LAN)

enable
configure terminal
hostname SW0

interface vlan1
ip address 192.168.1.2 255.255.255.0
no shutdown

ip default-gateway 192.168.1.1

logging 192.168.1.20

end
write

5️⃣ Switch1 Configuration (PC LAN)

enable
configure terminal
hostname SW1

interface vlan1
ip address 192.168.3.2 255.255.255.0
no shutdown

ip default-gateway 192.168.3.1

logging 192.168.1.20

end
write

6️⃣ NTP Server Configuration

On Server (192.168.1.10) in Packet Tracer:

Services → NTP → ON

IP configuration:

IP Address: 192.168.1.10
Mask: 255.255.255.0
Gateway: 192.168.1.1

7️⃣ Syslog Server Configuration

On Server (192.168.1.20):

Services → SYSLOG → ON

IP configuration:

IP Address: 192.168.1.20
Mask: 255.255.255.0
Gateway: 192.168.1.1

8️⃣ PC Configuration

PC0:

IP: 192.168.3.10
Mask: 255.255.255.0
Gateway: 192.168.3.1

9️⃣ Verification Commands

Check OSPF neighbors

show ip ospf neighbor

Check OSPF routes

show ip route ospf

Check NTP sync

show ntp associations

Check logging

show logging

Ping test

ping 192.168.1.10
ping 192.168.1.20

2.

Configure AAA Authentication

a.

Configure a local user account on Router and configure authenticate on the console

and vty lines using local AAA

b.

Verify local AAA authentication from the Router console and the PC-A client

1️⃣ Addressing Plan (Based on Your Diagram)

| Device | Interface | IP Address |
| --- | --- | --- |
| Router0 | G0/0 | 198.162.1.1 /24 |
| Router0 | S0/1/0 | 192.168.3.1 /30 |
| Router1 | G0/0 | 192.168.2.1 /24 |
| Router1 | S0/1/0 | 192.168.3.2 /30 |
| Router1 | S0/1/1 | 192.168.4.1 /30 |
| Router2 | G0/0 | 192.168.5.1 /24 |
| Router2 | S0/1/1 | 192.168.4.2 /30 |

PC Networks:

| Device | IP | Gateway |
| --- | --- | --- |
| PC0 | 192.168.2.10 | 192.168.2.1 |
| PC1 | 198.162.1.10 | 198.162.1.1 |
| PC2 | 192.168.5.10 | 192.168.5.1 |

2️⃣ Router0 Configuration

enable
configure terminal
hostname Router0

interface g0/0
ip address 198.162.1.1 255.255.255.0
no shutdown

interface s0/1/0
ip address 192.168.3.1 255.255.255.252
clock rate 64000
no shutdown

router ospf 1
network 198.162.1.0 0.0.0.255 area 0
network 192.168.3.0 0.0.0.3 area 0

aaa new-model

username admin privilege 15 secret cisco123

aaa authentication login default local

ip domain-name lab.local

crypto key generate rsa
1024

ip ssh version 2

line console 0
login authentication default
exit

line vty 0 4
login authentication default
transport input telnet ssh
exit

end
write memory

3️⃣ Router1 Configuration

enable
configure terminal
hostname Router1

interface g0/0
ip address 192.168.2.1 255.255.255.0
no shutdown

interface s0/1/0
ip address 192.168.3.2 255.255.255.252
no shutdown

interface s0/1/1
ip address 192.168.4.1 255.255.255.252
clock rate 64000
no shutdown

router ospf 1
network 192.168.2.0 0.0.0.255 area 0
network 192.168.3.0 0.0.0.3 area 0
network 192.168.4.0 0.0.0.3 area 0

aaa new-model

username admin privilege 15 secret cisco123

aaa authentication login default local

ip domain-name lab.local

crypto key generate rsa
1024

ip ssh version 2

line console 0
login authentication default
exit

line vty 0 4
login authentication default
transport input telnet ssh
exit

end
write memory

4️⃣ Router2 Configuration

enable
configure terminal
hostname Router2

interface g0/0
ip address 192.168.5.1 255.255.255.0
no shutdown

interface s0/1/1
ip address 192.168.4.2 255.255.255.252
no shutdown

router ospf 1
network 192.168.5.0 0.0.0.255 area 0
network 192.168.4.0 0.0.0.3 area 0

aaa new-model

username admin privilege 15 secret cisco123

aaa authentication login default local

ip domain-name lab.local

crypto key generate rsa
1024

ip ssh version 2

line console 0
login authentication default
exit

line vty 0 4
login authentication default
transport input telnet ssh
exit

end
write memory

5️⃣ Switch Configuration (Optional)

enable
configure terminal
hostname Switch1

interface vlan1
ip address 198.162.1.2 255.255.255.0
no shutdown

ip default-gateway 198.162.1.1
end
write

(Repeat similar config for other switches with their subnet.)

6️⃣ PC Configuration

PC0

IP: 192.168.2.10
Mask: 255.255.255.0
Gateway: 192.168.2.1

PC1

IP: 198.162.1.10
Mask: 255.255.255.0
Gateway: 198.162.1.1

PC2

IP: 192.168.5.10
Mask: 255.255.255.0
Gateway: 192.168.5.1

7️⃣ Verify AAA Authentication from Console

Disconnect console and reconnect.

Router will display:

User Access Verification

Username:
Password:

Enter:

Username: admin
Password: cisco123

8️⃣ Verify AAA from PC Client

On PC Command Prompt:

telnet 192.168.2.1

or

ssh -l admin 192.168.2.1

Output:

Username: admin
Password: cisco123
Router>

9️⃣ Verification Commands (Router)

Check AAA:

show running-config | include aaa

Check user:

show running-config | include username

Check SSH:

show ip ssh

Check OSPF neighbors:

show ip ospf neighbor