3.

Configuring Extended ACLs

a.

Configure, Apply and Verify an Extended Numbered ACL

1️⃣ Addressing Plan

| Network | Device | IP |
| --- | --- | --- |
| Server LAN | Server0 | 192.168.1.10 |
| Router G0/0 | 192.168.1.1 |  |
| PC0 LAN | PC0 | 192.168.2.10 |
| Router G0/1 | 192.168.2.1 |  |
| PC1 LAN | PC1 | 192.168.3.10 |
| Router G0/2 | 192.168.3.1 |  |

Subnet mask for all:

255.255.255.0

2️⃣ Router Configuration

Enter router CLI.

enable
configure terminal
hostname R1

Configure Interfaces

Server Network

interface g0/0
ip address 192.168.1.1 255.255.255.0
no shutdown
exit

PC0 Network

interface g0/1
ip address 192.168.2.1 255.255.255.0
no shutdown
exit

PC1 Network

interface g0/2
ip address 192.168.3.1 255.255.255.0
no shutdown
exit

3️⃣ PC and Server Configuration

PC0

IP: 192.168.2.10
Mask: 255.255.255.0
Gateway: 192.168.2.1

PC1

IP: 192.168.3.10
Mask: 255.255.255.0
Gateway: 192.168.3.1

Server

IP: 192.168.1.10
Mask: 255.255.255.0
Gateway: 192.168.1.1

4️⃣ Configure Extended ACL

Example requirement (common in labs):

👉 Block PC0 (192.168.2.10) from accessing Server (192.168.1.10)
👉 Allow all other traffic.

Create Extended ACL 100

access-list 100 deny ip host 192.168.2.10 host 192.168.1.10
access-list 100 permit ip any any

5️⃣ Apply ACL to Interface

Apply ACL closest to source network (PC0 network)

interface g0/1
ip access-group 100 in
exit

6️⃣ Save Configuration

end
write memory

7️⃣ Verify ACL

Check ACL list

show access-lists

Example output:

Extended IP access list 100
deny ip host 192.168.2.10 host 192.168.1.10
permit ip any any

Check interface ACL

show ip interface g0/1

You should see:

Inbound access list is 100

8️⃣ Test Connectivity

From PC0

ping 192.168.1.10

Result:

Request timed out

🚫 Blocked by ACL

From PC1

ping 192.168.1.10

Result:

Reply from 192.168.1.10

✅ Allowed

9️⃣ Test Router Connectivity

show access-lists

Hit counter example:

deny ip host 192.168.2.10 host 192.168.1.10 (5 matches)

Layer 2 VLAN Security

1️⃣ VLAN Plan

| VLAN | Name | Devices |
| --- | --- | --- |
| VLAN 10 | SALES | PC0, PC7 |
| VLAN 20 | HR | PC1 |
| VLAN 5 | IT | PC2, PC3, PC4, PC5 |
| VLAN 99 | MANAGEMENT | PC6 |

Network example:

| VLAN | Network |
| --- | --- |
| VLAN10 | 192.168.10.0/24 |
| VLAN20 | 192.168.20.0/24 |
| VLAN5 | 192.168.5.0/24 |
| VLAN99 | 192.168.99.0/24 |

2️⃣ Core Switch (Middle Switch)

enable
configure terminal
hostname CoreSwitch

Create VLANs

vlan 10
name SALES
exit

vlan 20
name HR
exit

vlan 5
name IT
exit

vlan 99
name MANAGEMENT
exit

Configure Trunk Ports

Ports connecting to other switches:

interface fa0/2
switchport mode trunk
switchport trunk allowed vlan 5,10,20,99
exit

interface fa0/3
switchport mode trunk
switchport trunk allowed vlan 5,10,20,99
exit

Router Connection

interface fa0/1
switchport mode trunk
exit

Security (Disable unused ports)

interface range fa0/4-24
shutdown
exit

3️⃣ Left Access Switch

enable
configure terminal
hostname Switch1

VLAN Creation

vlan 10
vlan 20
vlan 5
vlan 99

Access Ports

PC0 → VLAN10

interface fa0/5
switchport mode access
switchport access vlan 10
switchport port-security
switchport port-security maximum 1
switchport port-security violation shutdown
switchport port-security mac-address sticky
exit

PC1 → VLAN20

interface fa0/2
switchport mode access
switchport access vlan 20
switchport port-security
switchport port-security mac-address sticky
exit

PC2 → VLAN5

interface fa0/3
switchport mode access
switchport access vlan 5
switchport port-security
switchport port-security mac-address sticky
exit

Management PC → VLAN99

interface fa0/1
switchport mode access
switchport access vlan 99
exit

Trunk to Core

interface fa0/4
switchport mode trunk
switchport trunk allowed vlan 5,10,20,99
exit

4️⃣ Right Access Switch

enable
configure terminal
hostname Switch2

VLAN Creation

vlan 10
vlan 5
vlan 99

Access Ports

PC3

interface fa0/2
switchport mode access
switchport access vlan 5
switchport port-security
switchport port-security mac-address sticky
exit

PC4

interface fa0/3
switchport mode access
switchport access vlan 5
switchport port-security
switchport port-security mac-address sticky
exit

PC5

interface fa0/4
switchport mode access
switchport access vlan 5
switchport port-security
switchport port-security mac-address sticky
exit

PC7 → VLAN10

interface fa0/5
switchport mode access
switchport access vlan 10
switchport port-security
switchport port-security mac-address sticky
exit

Trunk to Core

interface fa0/1
switchport mode trunk
switchport trunk allowed vlan 5,10,20,99
exit

5️⃣ Router Configuration (Router-on-a-Stick)

enable
configure terminal
hostname Router0

Enable interface

interface g0/0
no shutdown

VLAN 10

interface g0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0
exit

VLAN 20

interface g0/0.20
encapsulation dot1Q 20
ip address 192.168.20.1 255.255.255.0
exit

VLAN 5

interface g0/0.5
encapsulation dot1Q 5
ip address 192.168.5.1 255.255.255.0
exit

Management VLAN

interface g0/0.99
encapsulation dot1Q 99
ip address 192.168.99.1 255.255.255.0
exit

6️⃣ PC IP Configuration

| PC | VLAN | IP | Gateway |
| --- | --- | --- | --- |
| PC0 | 10 | 192.168.10.10 | 192.168.10.1 |
| PC1 | 20 | 192.168.20.10 | 192.168.20.1 |
| PC2 | 5 | 192.168.5.10 | 192.168.5.1 |
| PC3 | 5 | 192.168.5.11 | 192.168.5.1 |
| PC4 | 5 | 192.168.5.12 | 192.168.5.1 |
| PC5 | 5 | 192.168.5.13 | 192.168.5.1 |
| PC7 | 10 | 192.168.10.11 | 192.168.10.1 |
| PC6 | 99 | 192.168.99.10 | 192.168.99.1 |

7️⃣ Verification Commands

Check VLAN

show vlan brief

Check trunk

show interfaces trunk

Check port security

show port-security

Check router VLAN interfaces

show ip interface brief