10.

Configuring ASA Basic Settings and Firewall Using CLI

a.

Configure basic ASA settings and interface security levels using CLI

b.

Configure routing, address translation, and inspection policy using CLI

c.

Configure DHCP, AAA, and SSH

d.

Configure a DMZ, Static NAT, and ACLs

| Network | Purpose |
| --- | --- |
| 155.0.1.0/24 | PC1 LAN (Right side) |
| 155.0.2.0/30 | R1–R2 serial |
| 155.0.3.0/30 | R0–R1 serial |
| 155.0.4.0/24 | R0–ASA (outside) |
| 155.0.5.0/24 | Server LAN (DMZ) |
| 155.0.6.0/24 | PC0 LAN (inside) |

🔹 STEP 1 — Configure Router0 (Left Router)

enable
configure terminal
hostname R0

interface g0/0
 ip address 155.0.4.1 255.255.255.0
 no shutdown

interface s0/1/0
 ip address 155.0.3.1 255.255.255.252
 clock rate 64000
 no shutdown

exit

ip route 155.0.1.0 255.255.255.0 155.0.3.2
ip route 155.0.6.0 255.255.255.0 155.0.4.2
ip route 155.0.5.0 255.255.255.0 155.0.4.2

end
write memory

🔹 STEP 2 — Configure Router1 (Top Router)

enable
configure terminal
hostname R1

interface s0/1/1
 ip address 155.0.3.2 255.255.255.252
 no shutdown

interface s0/1/0
 ip address 155.0.2.1 255.255.255.252
 clock rate 64000
 no shutdown

exit

ip route 155.0.4.0 255.255.255.0 155.0.3.1
ip route 155.0.6.0 255.255.255.0 155.0.3.1
ip route 155.0.5.0 255.255.255.0 155.0.3.1
ip route 155.0.1.0 255.255.255.0 155.0.2.2

end
write memory

🔹 STEP 3 — Configure Router2 (Right Router)

enable
configure terminal
hostname R2

interface g0/0
 ip address 155.0.1.1 255.255.255.0
 no shutdown

interface s0/1/0
 ip address 155.0.2.2 255.255.255.252
 no shutdown

exit

ip route 155.0.4.0 255.255.255.0 155.0.2.1
ip route 155.0.6.0 255.255.255.0 155.0.2.1
ip route 155.0.5.0 255.255.255.0 155.0.2.1

end
write memory

🔹 STEP 4 — Configure ASA 5505 (Main Part 🔥)

We will configure:

outside → 155.0.4.2

inside → 155.0.6.1

dmz → 155.0.5.1

NAT

Default route

ACL

🔸 Enter ASA CLI

enable
configure terminal
hostname ASA1

🔸 Configure Interfaces

interface vlan 1
 nameif inside
 security-level 100
 ip address 155.0.6.1 255.255.255.0
 no shutdown

interface vlan 2
 nameif outside
 security-level 0
 ip address 155.0.4.2 255.255.255.0
 no shutdown

interface vlan 3
 nameif dmz
 security-level 50
 ip address 155.0.5.1 255.255.255.0
 no shutdown

🔸 Assign Physical Ports

interface ethernet0/0
 switchport access vlan 2

interface ethernet0/1
 switchport access vlan 1

interface ethernet0/2
 switchport access vlan 3

🔸 Configure Default Route

route outside 0.0.0.0 0.0.0.0 155.0.4.1

🔸 Configure PAT (Inside to Outside Internet)

object network INSIDE-NET
 subnet 155.0.6.0 255.255.255.0
 nat (inside,outside) dynamic interface

🔸 Configure Static NAT for Server (DMZ to Outside)

object network DMZ-SERVER
 host 155.0.5.10
 nat (dmz,outside) static 155.0.4.10

🔸 Allow Outside Access to Server (HTTP Example)

access-list OUTSIDE-IN permit tcp any host 155.0.4.10 eq 80
access-group OUTSIDE-IN in interface outside

🔸 Save Configuration

write memory

🔹 STEP 5 — Configure PC0 (Inside LAN)

Desktop → IP Configuration

IP Address: 155.0.6.10
Subnet Mask: 255.255.255.0
Default Gateway: 155.0.6.1

🔹 STEP 6 — Configure Server (DMZ)

Desktop → IP Configuration

IP Address: 155.0.5.10
Subnet Mask: 255.255.255.0
Default Gateway: 155.0.5.1

Enable HTTP service:

Services → HTTP → ON

🔹 STEP 7 — Configure PC1 (Right LAN)

IP Address: 155.0.1.10
Subnet Mask: 255.255.255.0
Default Gateway: 155.0.1.1

🔥 TESTING

From PC0:

ping 155.0.1.10
ping 155.0.5.10

From PC1:

ping 155.0.4.10

Browser test:

http://155.0.4.10

Configure and Verify a Site-to-Site IPsec VPN Using CLI

1️⃣ Addressing Plan

| Device | Interface | IP |
| --- | --- | --- |
| Router1 | G0/0 | 192.168.1.1 /24 |
| Router1 | S0/1/0 | 192.168.4.2 /30 |
| Router0 (ISP) | S0/1/0 | 192.168.4.1 /30 |
| Router0 | S0/1/1 | 192.168.5.1 /30 |
| Router0 | G0/0 | 192.168.2.1 /24 |
| Router2 | S0/1/0 | 192.168.5.2 /30 |
| Router2 | G0/0 | 192.168.3.1 /24 |

PCs:

| PC | IP | Gateway |
| --- | --- | --- |
| PC1 | 192.168.1.10 | 192.168.1.1 |
| PC0 | 192.168.2.10 | 192.168.2.1 |
| PC2 | 192.168.3.10 | 192.168.3.1 |

2️⃣ Router1 Configuration (Left Site)

enable
conf t
hostname Router1

interface g0/0
ip address 192.168.1.1 255.255.255.0
no shutdown

interface s0/1/0
ip address 192.168.4.2 255.255.255.252
no shutdown

Default Route to ISP

ip route 0.0.0.0 0.0.0.0 192.168.4.1

Phase 1 – ISAKMP Policy

crypto isakmp policy 10
encryption aes
hash sha
authentication pre-share
group 2
lifetime 86400
exit

Pre-shared key

crypto isakmp key cisco address 192.168.5.2

Phase 2 – Transform Set

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac
mode tunnel

Interesting Traffic (VPN ACL)

access-list 110 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255

Crypto Map

crypto map VPN-MAP 10 ipsec-isakmp
set peer 192.168.5.2
set transform-set VPN-SET
match address 110

Apply Crypto Map

interface s0/1/0
crypto map VPN-MAP
exit

3️⃣ Router2 Configuration (Right Site)

enable
conf t
hostname Router2

interface g0/0
ip address 192.168.3.1 255.255.255.0
no shutdown

interface s0/1/0
ip address 192.168.5.2 255.255.255.252
no shutdown

Default Route

ip route 0.0.0.0 0.0.0.0 192.168.5.1

Phase 1

crypto isakmp policy 10
encryption aes
hash sha
authentication pre-share
group 2
lifetime 86400
exit

Pre-shared Key

crypto isakmp key cisco address 192.168.4.2

Phase 2

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac
mode tunnel

VPN ACL

access-list 110 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255

Crypto Map

crypto map VPN-MAP 10 ipsec-isakmp
set peer 192.168.4.2
set transform-set VPN-SET
match address 110

Apply Crypto Map

interface s0/1/0
crypto map VPN-MAP
exit

4️⃣ ISP Router Configuration (Router0)

enable
conf t
hostname ISP

interface g0/0
ip address 192.168.2.1 255.255.255.0
no shutdown

interface s0/1/0
ip address 192.168.4.1 255.255.255.252
clock rate 64000
no shutdown

interface s0/1/1
ip address 192.168.5.1 255.255.255.252
clock rate 64000
no shutdown

Static Routes

ip route 192.168.1.0 255.255.255.0 192.168.4.2
ip route 192.168.3.0 255.255.255.0 192.168.5.2

5️⃣ Verification Commands

Check ISAKMP Phase 1

show crypto isakmp sa

Expected:

QM_IDLE

Check IPsec Phase 2

show crypto ipsec sa

Check Crypto Map

show crypto map

6️⃣ Test VPN Tunnel

Ping from PC1 → PC2

ping 192.168.3.10

First ping may fail, next should succeed.

7️⃣ Verify Encrypted Packets

On Router1:

show crypto ipsec sa

You should see:

pkts encrypt
pkts decrypt