!!!Script for Central

conf t

spanning-tree vlan 1 root primary

end

!!!Script for SW-1 conf t

spanning-tree vlan 1 root secondary

interface range f0/23 – 24

spanning-tree guard root

end

!!!Script for SW-2 conf t

interface range f0/23 – 24

spanning-tree guard root

end

!!!Script for SW-A conf t

interface range f0/1 – 4

spanning-tree portfast

spanning-tree bpduguard enable

interface range f0/1 – 22

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security mac-address sticky

interface range f0/5 – 22

shutdown

end

!!!Script for SW-B conf t

interface range f0/1 - 4

spanning-tree portfast

spanning-tree bpduguard enable

interface range f0/1 – 22

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security violation

shutdown

switchport port-security mac-address sticky

interface range f0/5 – 22

shutdown

end